Homojen dolgulu sıkıştırma ile ateşlemeli bir motorda aseton bütanol etanol üçlü yakıt karışımlarının çalışma aralığına motor performansına ve emisyonlara etkisinin incelenmesi

İsimli GAP Projesi için Teknik Şartname Dosyası

PROJE ID: 8566

SARF MALZEMELER TEKNİK ŞARTNAME

N-bütanol (1-bütanol):

a) Physical state liquid, clear

b) Color colorless

c) Odor ethanolic

d) Melting point/range: -90 °C

e) Initial boiling point and boiling range 116 - 118 °C

f) Purity: ≥%99.5

g) Upper/lower flammability or explosive limits

Upper explosion limit: 11,2 %(V)

Lower explosion limit: 1,4 %(V)

h) Flash point 35 °C - Pensky-Martens closed cup - ISO 2719

i) pH 7 at 70 g/l at 20 °C

j) Viscosity, dynamic: 2,95 mPa.s at 20 °C

k) Water solubility 66 g/l at 20 °C - OECD Test Guideline 105

l) Partition coefficient: log Pow: 1 at 25 °C - Bioaccumulation is not expected.

m) Vapor pressure < 10 hPa at 20 °C

n) Density 0,81 g/mL at 25 °C

N-heptan:

a) Physical state liquid

b) Color colorless

c) Purity: ≥%99.5

d) Melting point/freezing point Melting point: -91,0 °C

e) Initial boiling point and boiling range 98,2 - 98,4 °C at 1.000 hPa

f) Upper/lower flammability or explosive limits

Upper explosion limit: 7 %(V)

Lower explosion limit: 1,1 %(V)

g) Flash point -4 °C - c.c.

h) Autoignition temperature 223,0 °C

i) Viscosity Viscosity, kinematic: 0,64 mm2/s at 20 °C

j) Water solubility insoluble

k) Partition coefficient log Pow: > 3 - Bioaccumulation is not expected.

l) Vapor pressure 48 hPa at 20,0 °C

m) Density 0,68 g/cm3 at 15 °C